Impulse and Momentum

Duy Nguyen, Brayan Hernandez, Ryan Dunford

Tyrone Clinton

PHY1048L – 2230

Oct 21, 2026

# Lab 8: Impulse and Momentum

PHY 1048L								Physics Laboratory I

Preliminary Questions:

1. In a car collision, the driver’s body must change speed from a high value to zero. This is true whether or not an airbag is used, so why use an airbag? How does it reduce injuries?

2. You want to close an open door by throwing either a 400 𝑔 lump of clay or a 400 𝑔 rubber ball toward it. You can throw either object with the same speed, but they are different in that the rubber ball bounces off the door while the clay just sticks to the door. Which projectile will apply the larger impulse to the door and be more likely to close it?

Data Table:

Figure 1—Mass of the Cart

| Mass of Cart (kg) | 0.5083 |
| --- | --- |

Table 1

| Trial | Final Velocity vf | Initial Velocity vi | Average Force F | Duration of Impulse t |
| --- | --- | --- | --- | --- |
| Elastic 1 | (m/s) | (m/s) | (N) | (s) |
| 1 | 0.4593 | -0.6051 | 1.354 | 0.467 |
| 2 | 0.5076 | -0.709 | 1.532 | 0.418 |
| Elastic 2 | (m/s) | (m/s) | (N) | (s) |
| 1 | 0.7931 | -0.7898 | 1.753 | 0.771 |
| 2 | 1.049 | -1.255 | 1.591 | 0.766 |

Summary and Analysis:

- From the average force and duration of impulse, determine:	Impulse = FaveΔt

Elastic 1:

1. 1.354×0.467=(J)

2. 1.532×0.418=(J)

Elastic 2:

1. 1.753×0.771=(J)

2. 1.591×0.766=(J)

- From the mass of the cart and change in velocity, determine: Change in momentum =∆p=mvf-mvi

Elastic 1:

1. 0.5083×[0.4593--0.6051]= (kg⋅m/s)

2. 0.5083×[0.5076--0.709]= (kg⋅m/s)

Elastic 2:

1. 0.5083×[0.7931--0.7898]= (kg⋅m/s)

2. 0.5083×[1.049--1.255]= (kg⋅m/s)

Table 2

| Trial | Impulse FaveΔt (J) | Change in momentum mvf-vi (kg⋅m/s) | % Difference between Impulse and Change in Momentum (%) |
| --- | --- | --- | --- |
| Elastic 1 |  |  |  |
| 1 |  |  |  |
| 2 |  |  |  |
| Elastic 2 | (J) | (kg⋅m/s) | (%) |
| 1 |  |  |  |
| 2 |  |  |  |

- If the impulse-momentum theorem is correct, the change in momentum will equal the impulse for each trial. Experimental measurement errors, along with friction and shifting of the track or Force Sensor, will keep the two from being exactly the same. One way to compare the two is to find their percentage difference using:

%difference= x1-x20.5(x1+x2)×100

Where:
x1 = impulse = FaveΔt 
x2= change in momentum =mvf-mvi

Elastic 1:

1. x1-x20.5(x1+x2)×100= %

2.x1-x20.5(x1+x2)×100=%

Elastic 2:

1. x1-x20.5(x1+x2)×100=%

2. x1-x20.5(x1+x2)×100=%

- Does your data support the impulse-momentum theorem? Explain.

- Look at the shape of the last force vs. time graph. Is there a way you could deliver the same impulse with a much smaller force? Explain.

6. When you used a stiffer or tighter elastic material, what effect did this have on the duration of the impulse?

(a)	What effect did this have on the maximum size of the force?

(b)	Can you develop a general rule from these observations?

- Give one random error for this experiment. Say explicitly why it is a random error.

- Give one systematic error for this experiment. Say explicitly why it is a systematic error.